Vast.ai Deployment Guide

Running AutoGluon (09C) and VAE (08B) on a Cloud GPU

Master Thesis Pipeline  —  Tristan Leiter  —  WU Wien

# Overview

Your HP Envy (i7-1065G7, 4 cores, integrated graphics, 15.8 GB RAM) is a laptop CPU from 2019 with no dedicated GPU. AutoGluon on this machine runs entirely on CPU with no parallelism, which is why training is slow.

A Vast.ai RTX 3090 instance (24 GB VRAM, 10,496 CUDA cores) will be roughly 20–50x faster for this workload. All 6 Phase 1 models can be completed in a single session of 3–4 hours at approximately €0.30/hr.

|  | Specification |
| --- | --- |
| Local machine | i7-1065G7, 4 cores, 1.5 GHz, 15.8 GB RAM, no GPU |
| Vast.ai target | RTX 3090, 24 GB VRAM, 64+ GB RAM, €0.25–0.35/hr |
| Speedup estimate | 20–50× vs local CPU |
| Phase 1 total cost | €2–3 for all 6 non-VAE models |
| Shell to use | Git Bash (Windows) — not CMD, not PowerShell |

# Step 0: One Code Change Before Uploading

Replace the DATA_ROOT line at the top of 09C_AutoGluon.py (and 08B_Autoencoder.py) with this OS-aware block. This is the only code change required:

import os

from pathlib import Path

# Automatically detect Windows (local) vs Linux (Vast.ai)

if os.name == 'nt':

DATA_ROOT = Path(r'C:\Users\Tristan Leiter\Documents\MT')

else:

DATA_ROOT = Path('/workspace/MT')   # standard Vast.ai directory

DIR_DATA     = DATA_ROOT / '02_Data'

DIR_FEATURES = DIR_DATA  / 'Features'

DIR_LABELS   = DIR_DATA  / 'Labels'

DIR_OUTPUT   = DATA_ROOT / '03_Output'

DIR_MODELS   = DIR_OUTPUT / 'Models' / 'AutoGluon'

DIR_TABLES   = DIR_OUTPUT / 'Tables'

Note: Apply the identical change to 08B_Autoencoder.py for the VAE runs.

# Step 1: Install the Vast.ai CLI (Once, on Your Windows Machine)

Open Git Bash and run:

pip install vastai

Then go to vast.ai → Account → CLI, copy your API key command, and run it in Git Bash:

vastai set api-key YOUR_KEY_HERE

# Step 2: Find and Launch an Instance

Search for a reliable RTX 3090 with fast internet:

vastai search offers "gpu_name=RTX_3090 num_gpus=1 reliability>0.99 inet_down>300 disk_space>=80" -o "dph"

Note the Instance ID from the leftmost column of the results. Then launch:

vastai create instance INSTANCE_ID \

--image nvcr.io/nvidia/pytorch:24.01-py3 \

--disk 80 \

--ssh \

--direct

Wait 2 minutes, then confirm it is running:

vastai show instances

# Step 3: Get the SSH Connection Details

vastai ssh-url INSTANCE_ID

This returns something like:  ssh root@123.45.67.89 -p 12345

Note the IP address and port number — you will use these in the next steps.

# Step 4: Upload Your Project (Git Bash)

Use rsync, not the vastai copy command (rsync is more reliable for large transfers). Run this from Git Bash on your Windows machine:

rsync -avz --progress \

-e "ssh -p 12345" \

"/c/Users/Tristan Leiter/Documents/MT/" \

root@123.45.67.89:/workspace/MT/

This transfers everything: RDS feature files, parquets, label files, Python scripts. Expect 5–15 minutes depending on your upload speed. This only needs to happen once per instance.

Note: After the initial upload, if you only change Python scripts (not data), you can re-run the same rsync command and it will only transfer the changed files.

# Step 5: SSH In and Install Dependencies

Connect to the instance:

ssh root@123.45.67.89 -p 12345

Then on the instance (Linux terminal), install the required packages:

pip install autogluon pyreadr pyarrow pandas numpy scikit-learn

AutoGluon installation takes 8–12 minutes. Do this once per instance. Then verify:

python -c "import autogluon; import pyreadr; print('OK')"

# Step 6: Switch the MODEL and Run 09C

The MODEL variable at the top of Section 2 in 09C_AutoGluon.py controls which model runs. Change it before each run.

## Option A: Edit the file on the instance (simplest)

Use the nano text editor directly on the instance:

nano /workspace/MT/01_Code/09C_AutoGluon.py

Navigate to the line that says MODEL = "..." (near the top, Section 2). Change the value. Then:

- Ctrl + O to save

- Enter to confirm filename

- Ctrl + X to exit

Then run the script:

cd /workspace/MT/01_Code

python 09C_AutoGluon.py

## Option B: One-line sed command (faster for sequential runs)

Use sed to replace the MODEL line without opening an editor:

# Change MODEL to M3

sed -i 's/^MODEL = .*/MODEL = "raw"/' /workspace/MT/01_Code/09C_AutoGluon.py

# Verify the change took effect

grep '^MODEL' /workspace/MT/01_Code/09C_AutoGluon.py

# Run

python /workspace/MT/01_Code/09C_AutoGluon.py

## Option C: Loop over all Phase 1 models automatically

This runs all 6 Phase 1 models back to back without any manual intervention:

cd /workspace/MT/01_Code

for model in fund raw bucket bucket_raw structural structural_raw; do

echo ""

echo "====================================="

echo "Starting model: $model"

echo "====================================="

sed -i "s/^MODEL = .*/MODEL = \"$model\"/" 09C_AutoGluon.py

python 09C_AutoGluon.py

done

echo "All Phase 1 models complete."

Note: With TIME_LIMIT = 1800 (30 min) per model, Phase 1 takes roughly 3.5 hours. Consider increasing TIME_LIMIT to 3600 on the GPU instance since the full ensemble can actually complete within that window, producing a stronger model.

## Model Keys Quick Reference

| Model | Key | Features | Label | Shift | OOS end | Phase |
| --- | --- | --- | --- | --- | --- | --- |
| M1 | fund | features_fund.rds | y_next (CSI) | lead(y,1) | 2024 | Phase 1 |
| M2 | latent_fund | features_latent_fund.parquet | y_next (CSI) | lead(y,1) | 2024 | Phase 2 |
| M3 | raw | features_raw.rds | y_next (CSI) | lead(y,1) | 2024 | Phase 1 |
| M4 | latent_raw | features_latent_raw.parquet | y_next (CSI) | lead(y,1) | 2024 | Phase 2 |
| B1 | bucket | features_fund.rds | y_loser | None | 2019 | Phase 1 |
| B2 | bucket_latent_fund | features_latent_fund.parquet | y_loser | None | 2019 | Phase 2 |
| B3 | bucket_raw | features_raw.rds | y_loser | None | 2019 | Phase 1 |
| B4 | bucket_latent_raw | features_latent_raw.parquet | y_loser | None | 2019 | Phase 2 |
| S1 | structural | features_fund.rds | y_structural | None | 2019 | Phase 1 |
| S2 | structural_latent_fund | features_latent_fund.parquet | y_structural | None | 2019 | Phase 2 |
| S3 | structural_raw | features_raw.rds | y_structural | None | 2019 | Phase 1 |
| S4 | structural_latent_raw | features_latent_raw.parquet | y_structural | None | 2019 | Phase 2 |

# Step 7: Run 08B (VAE) for Phase 2

Phase 2 models (M2, M4, B2, B4, S2, S4) require the VAE latent features to exist first. Run 08B twice:

# First run: VAE on fundamentals (produces features_latent_fund.parquet)

sed -i 's/^VAE_INPUT = .*/VAE_INPUT = "fund"/' /workspace/MT/01_Code/08B_Autoencoder.py

python /workspace/MT/01_Code/08B_Autoencoder.py

# Second run: VAE on full features (produces features_latent_raw.parquet)

sed -i 's/^VAE_INPUT = .*/VAE_INPUT = "raw"/' /workspace/MT/01_Code/08B_Autoencoder.py

python /workspace/MT/01_Code/08B_Autoencoder.py

Then run Phase 2 models using the same loop pattern as Phase 1:

for model in latent_fund latent_raw bucket_latent_fund bucket_latent_raw structural_latent_fund structural_latent_raw; do

sed -i "s/^MODEL = .*/MODEL = \"$model\"/" /workspace/MT/01_Code/09C_AutoGluon.py

python /workspace/MT/01_Code/09C_AutoGluon.py

done

# Step 8: Download Results

From Git Bash on your Windows machine, download all outputs:

rsync -avz --progress \

-e "ssh -p 12345" \

root@123.45.67.89:/workspace/MT/03_Output/ \

"/c/Users/Tristan Leiter/Documents/MT/03_Output/"

Verify the parquet files exist locally before destroying the instance:

ls "/c/Users/Tristan Leiter/Documents/MT/03_Output/Tables/ag_fund/"

# Expected: ag_preds_test_eval.parquet, ag_preds_test.parquet,

#           ag_preds_oos.parquet, ag_leaderboard.csv, ag_eval_summary.json

# Step 9: Destroy the Instance

You stop paying immediately when the instance is destroyed. Run this only after confirming all results have downloaded correctly:

vastai destroy instance INSTANCE_ID

Do not use stop (paused instances still incur storage charges). Destroy terminates billing completely.

# Time and Cost Estimates

|  | Details |
| --- | --- |
| Phase 1 (6 models) | M1, M3, B1, B3, S1, S3 — ~35 min each incl. CV — ~3.5 hrs total |
| 08B VAE (2 runs) | fund + raw — ~1 hr each — ~2 hrs total |
| Phase 2 (6 models) | M2, M4, B2, B4, S2, S4 — ~30 min each — ~3 hrs total |
| Total compute | ~8–9 hours across two sessions |
| Cost at €0.30/hr | ~€2.50 Phase 1 ¦ ~€3.00 Phase 2 ¦ ~€6 total |
| TIME_LIMIT setting | Consider 3600s (60 min) on GPU vs 1800s locally — full ensemble completes on GPU |

Note: Keep Phase 1 and Phase 2 as separate Vast.ai sessions if desired. Download and verify Phase 1 results, destroy the instance, then start a fresh one for 08B + Phase 2.

# Troubleshooting

| Issue | Fix |
| --- | --- |
| rsync: command not found | Use Git Bash, not CMD or PowerShell |
| SSH connection refused | Wait 2 more minutes after create; re-run vastai show instances |
| ModuleNotFoundError pyreadr | pip install pyreadr (re-run on instance) |
| FileNotFoundError: features | Check DATA_ROOT os.name block was applied; verify rsync completed |
| CUDA out of memory | Reduce batch size in 08B CFG, or choose instance with more VRAM |
| Instance disappeared | Host may have reclaimed it; re-upload and re-run from last checkpoint |
| pip install autogluon fails | Try: pip install autogluon --no-deps then pip install autogluon |

All commands run in Git Bash (Windows). The instance IP and port change each time you create a new instance — run vastai ssh-url INSTANCE_ID to retrieve them.